Министерство образования Республики Беларусь

Учреждение образования

«Белорусский государственный университет информатики и радиоэлектроники»

Кафедра электронных вычислительных машин

Лабораторная работа №3

«Программирование системного таймера»

Вариант 10

Выполнил:									Проверил:

Студент группы 450502						Преподаватель

Новицкий Е.Ю.								Марзалюк А.В.

Минск, 2026

# Постановка задачи

Запрограммировать второй канал таймера таким образом, чтобы динамик компьютера издавал звуки.

Для всех каналов таймера считать слово состояния и вывести его на экран в двоичной форме.

# Алгоритм

Для того чтобы динамик компьютера издавал звуки, необходимо выполнить следующие действия:

- Вывести в порт управляющего регистра с адресом 43h управляющее слово 10110110, соответствующее каналу 2, режиму 3

- Установить значение счётчика канала 2 таймера: в порт 42h вывести значение, полученное при разделении 1193180 на требуемую частоту в герцах, причём вначале вывести младший, а затем старший байты.

- Установить в 1 два младших бита порта 61h для включения звука. Для этого вначале считывается байт из порта 61h в рабочую ячейку памяти, устанавливаются нужные биты, затем выводится новое значение байта в порт 61h.

- Установить в 0 два младших бита порта 61h для выключения звука.

Для чтения слова состояния каналов необходимо:

- Вывести в порт управляющего регистра с адресом 43h управляющее слово, соответствующее команде RBC (Чтение состояния канала) и номеру канала.

- Вывести из порта нужного канала слово состояния.

# Листинг программы

Далее приведен листинг программы, реализующей все поставленные задачи.

#include <stdio.h>

#include <dos.h>

#include <conio.h>

/* base frequency of system timer */

#define PIT_BASE_FREQUENCY 1193180UL

/* ports of system timer and system dynamic */

#define PIT_CHANNEL0_PORT 0x40

#define PIT_CHANNEL1_PORT 0x41

#define PIT_CHANNEL2_PORT 0x42

#define PIT_CONTROL_PORT  0x43

#define SPEAKER_PORT      0x61

typedef struct

{

unsigned int frequency;

unsigned int duration;

unsigned int pause;

} Note;

void setSpeakerFrequency(unsigned int frequency)

{

unsigned int divider;

unsigned char lowByte;

unsigned char highByte;

if (frequency == 0)

{

return;

}

divider = (unsigned int)(PIT_BASE_FREQUENCY / frequency);

lowByte = (unsigned char)(divider & 0xFF);

highByte = (unsigned char)((divider >> 8) & 0xFF);

outp(PIT_CONTROL_PORT, 0xB6);

outp(PIT_CHANNEL2_PORT, lowByte);

outp(PIT_CHANNEL2_PORT, highByte);

}

void speakerOn(void)

{

unsigned char value;

value = inp(SPEAKER_PORT);

value = value | 0x03;

outp(SPEAKER_PORT, value);

}

void speakerOff(void)

{

unsigned char value;

value = inp(SPEAKER_PORT);

value = value & 0xFC;

outp(SPEAKER_PORT, value);

}

void playNote(unsigned int frequency, unsigned int duration, unsigned int pause)

{

if (frequency == 0)

{

delay(duration);

delay(pause);

return;

}

setSpeakerFrequency(frequency);

speakerOn();

delay(duration);

speakerOff();

delay(pause);

}

void playTestMelody(void)

{

Note melody[] =

{

{440, 300, 100},

{494, 300, 100},

{523, 300, 100},

{587, 300, 100},

{659, 500, 150}

};

int i;

int count = sizeof(melody) / sizeof(melody[0]);

printf("Playing test melody...\n");

for (i = 0; i < count; i++)

{

playNote(melody[i].frequency, melody[i].duration, melody[i].pause);

}

}

void playMasterOfPuppets(void)

{

Note melody[] =

{

{329, 180, 40},

{329, 180, 40},

{329, 180, 40},

{311, 180, 40},

{329, 180, 40},

{392, 250, 60},

{293, 180, 40},

{293, 180, 40},

{293, 180, 40},

{277, 180, 40},

{293, 180, 40},

{349, 250, 60},

{329, 180, 40},

{329, 180, 40},

{392, 250, 60},

{440, 300, 80},

{0,   200, 0},    /* pause */

{392, 180, 40},

{349, 180, 40},

{329, 250, 60},

{293, 300, 80}

};

int i;

int count = sizeof(melody) / sizeof(melody[0]);

printf("Playing: Metallica - Master of Puppets\n");

for (i = 0; i < count; i++)

{

playNote(melody[i].frequency, melody[i].duration, melody[i].pause);

}

}

void printBinaryByte(unsigned char value)

{

int i;

for(i = 7; i >= 0; i--)

{

if(value & (1 << i))

{

printf("1");

}

else

{

printf("0");

}

}

}

void printStatusWords(void)

{

unsigned char status;

int ports[3] = {PIT_CHANNEL0_PORT, PIT_CHANNEL1_PORT, PIT_CHANNEL2_PORT};

unsigned char controlWords[3] = {0xE2, 0xE4, 0xE8};

int channel;

printf("\nStatus words of timer channels:\n");

for (channel = 0; channel < 3; channel++)

{

outp(PIT_CONTROL_PORT, controlWords[channel]);

status = inp(ports[channel]);

printf("Channel %d: ", channel);

printBinaryByte(status);

printf("\n");

}

}

unsigned int readCounterValue(int channelPort, unsigned char latchCommand)

{

unsigned char lowByte;

unsigned char highByte;

unsigned int value;

outp(PIT_CONTROL_PORT, latchCommand);

lowByte = inp(channelPort);

highByte = inp(channelPort);

value = ((unsigned int) highByte << 8) | lowByte;

return value;

}

void printCounterValues(void)

{

int ports[3] = {PIT_CHANNEL0_PORT, PIT_CHANNEL1_PORT, PIT_CHANNEL2_PORT};

unsigned char latchCommands[3] = {0x00, 0x40, 0x80};

unsigned int counterValue;

int channel;

printf("\nCounter values (CE) in hexadecimal:\n");

for (channel = 0; channel < 3; channel++)

{

counterValue = readCounterValue(ports[channel], latchCommands[channel]);

printf("Channel %d: %04Xh\n", channel, counterValue);

}

}

int showMenu(void)

{

int option;

printf("\n==============================\n");

printf("Programmable Interval Timer Lab\n");

printf("==============================\n");

printf("1. Play test melody\n");

printf("2. Play Metallica - Master of Puppets\n");

printf("3. Show status words\n");

printf("4. Show counter values (CE)\n");

printf("5. Exit\n");

printf("Choose option: ");

scanf("%d", &option);

return option;

}

int main(void)

{

int option;

clrscr();

while (1)

{

option = showMenu();

switch (option)

{

case 1:

clrscr();

playTestMelody();

break;

case 2:

clrscr();

playMasterOfPuppets();

break;

case 3:

clrscr();

printStatusWords();

break;

case 4:

clrscr();

printCounterValues();

break;

case 5:

clrscr();

speakerOff();

return 0;

default:

clrscr();

printf("Invalid option. Try again.\n");

break;

}

}

}

# Тестирование программы

Во время работы программы происходит звучание системного динамика. Также для всех каналов таймера выводится на экран в двоичной форме слово состояния:

# Рисунок 4.1 – Результат работы программы при выводе слов состояния каналов таймера.

# Заключение

В ходе лабораторной работы удалось запрограммировать второй канал таймера таким образом, чтобы динамик компьютера издавал звук, а также для всех каналов таймера было считано слово состояния и выведено на экран в двоичной форме.

Программа компилировалась в Turbo C++ и запускалась в DOS, который эмулировался с помощью DosBox-X.