Министерство образования Республики Беларусь

Учреждение образования

«Белорусский государственный университет информатики и радиоэлектроники»

Кафедра электронных вычислительных машин

Лабораторная работа №4

«Программирование часов реального времени»

Выполнил:									Проверил:

Студент группы 450502						Преподаватель

Новицкий Е.Ю.								Марзалюк А.В.

Минск, 2026

# Постановка задачи

Написать программу, которая будет считывать и устанавливать время в часах реального времени. Считанное время должно выводиться на экран в удобочитаемой форме.

- Используя аппаратное прерывание часов реального времени и режим генерации периодических прерываний реализовать функцию задержки с точностью в миллисекунды.

- Используя аппаратное прерывания часов реального времени и режим будильника реализовать функции программируемого будильника.

# Алгоритм

#### Перед установкой значений времени вызывается функция, которая считывает и анализирует старший байт регистра состояния 1 на предмет доступности значений для чтения и записи. Когда этот бит установлен в '0', отключается внутренний цикл обновления часов реального времени: для этого старший бит регистра состояния 2 устанавливается в '1'.

#### Считывание или запись значений времени происходит следующим образом: в порт 70h отправляется индекс регистра CMOS, соответствующий значению времени (секунды, часы и т. д.), затем происходит чтение значения из порта 71h (или запись значения в порт).

#### После установки значений времени вызывается функция, которая возобновляет внутренний цикл обновления часов реального времени.

#### Для реализации функции задержки заменён обработчик прерывания 0x70, в котором происходит отсчёт миллисекунд. Для включения периодического прерывания, происходящего примерно каждую миллисекунду, 6-й бит регистра B устанавливается в '1'.

# Листинг программы

Далее приведен листинг программы, реализующей все поставленные задачи.

#include <stdio.h>

#include <dos.h>

#include <conio.h>

#define RTC_INDEX_PORT 0x70 // choose

#define RTC_DATA_PORT  0x71 // read or write

// RTC registers

#define RTC_SECONDS      0x00

#define RTC_ALARM_SEC    0x01

#define RTC_MINUTES      0x02

#define RTC_ALARM_MIN    0x03

#define RTC_HOURS        0x04

#define RTC_ALARM_HOUR   0x05

#define RTC_WEEKDAY      0x06

#define RTC_DAY          0x07

#define RTC_MONTH        0x08

#define RTC_YEAR         0x09

#define RTC_STATUS_A     0x0A

#define RTC_STATUS_B     0x0B

#define RTC_STATUS_C     0x0C

#define RTC_STATUS_D     0x0D

#define RTC_CENTURY      0x32

// PIC

// rtc on irq8 – second pic

#define PIC1_COMMAND     0x20

#define PIC1_DATA        0x21

#define PIC2_COMMAND     0xA0

#define PIC2_DATA        0xA1

#define PIC_EOI          0x20

volatile unsigned long rtc_ticks = 0; // tick counter

volatile unsigned long target_ticks = 0; // ticks to wait

volatile int delay_done = 0; // flag of delay end

volatile int alarm_fired = 0; // flag of alarm worked

// old_handler

void interrupt (*old_rtc_handler)(void);

// old pic masks, to recover after

unsigned char old_pic1_mask = 0;

unsigned char old_pic2_mask = 0;

unsigned char read_rtc(unsigned char reg)

{

outp(RTC_INDEX_PORT, reg);

return inp(RTC_DATA_PORT);

}

void write_rtc(unsigned char reg, unsigned char value)

{

outp(RTC_INDEX_PORT, reg);

outp(RTC_DATA_PORT, value);

}

int bcd_to_int(unsigned char bcd)

{

return ((bcd >> 4) * 10) + (bcd & 0x0F);

}

unsigned char int_to_bcd(int value)

{

return (unsigned char)(((value / 10) << 4) | (value % 10));

}

void wait_rtc_ready(void)

{

while (read_rtc(RTC_STATUS_A) & 0x80) {}

}

int rtc_is_binary_mode(void)

{

unsigned char regB = read_rtc(RTC_STATUS_B);

return (regB & 0x04) ? 1: 0;

}

int rtc_is_24hour_mode(void)

{

unsigned char regB = read_rtc(RTC_STATUS_B);

return (regB & 0x02) ? 1: 0;

}

// read and set time

void show_current_time(void)

{

unsigned char sec, min, hour, day, month, year, century;

int isBinary;

wait_rtc_ready();

sec = read_rtc(RTC_SECONDS);

min = read_rtc(RTC_MINUTES);

hour = read_rtc(RTC_HOURS);

day = read_rtc(RTC_DAY);

month = read_rtc(RTC_MONTH);

year = read_rtc(RTC_YEAR);

century = read_rtc(RTC_CENTURY);

isBinary = rtc_is_binary_mode();

if (!isBinary)

{

sec = bcd_to_int(sec);

min = bcd_to_int(min);

hour = bcd_to_int(hour);

day = bcd_to_int(day);

month = bcd_to_int(month);

year = bcd_to_int(year);

century = bcd_to_int(century);

}

clrscr();

printf("========================================\n");

printf("       CURRENT RTC TIME AND DATE\n");

printf("========================================\n\n");

printf("Date : %02u.%02u.%02u%02u\n", day, month, century, year);

printf("Time : %02u:%02u:%02u\n", hour, min, sec);

printf("\nStorage format: %s\n", isBinary ? "binary" : "BCD");

printf("Hour mode     : %s\n", rtc_is_24hour_mode() ? "24-hour" : "12-hour");

printf("\nPress any key...");

getch();

}

int validate_datetime(int hour, int min, int sec, int day, int month, int year, int century)

{

if (hour < 0 || hour > 23) return 0;

if (min < 0 || min > 59) return 0;

if (sec < 0 || sec > 59) return 0;

if (day < 1 || day > 31) return 0;

if (month < 1 || month > 12) return 0;

if (year < 0 || year > 99) return 0;

if (century < 0 || century > 99) return 0;

return 1;

}

void set_current_time(void)

{

// check UIP

// set bit 7 regB

// write new values

// reset bit 7 reB

int hour, min, sec, day, month, year, century;

unsigned char regB;

int isBinary;

clrscr();

printf("========================================\n");

printf("        SET RTC TIME AND DATE\n");

printf("========================================\n\n");

printf("Enter hours      (0-23): ");

scanf("%d", &hour);

printf("Enter minutes    (0-59): ");

scanf("%d", &min);

printf("Enter seconds    (0-59): ");

scanf("%d", &sec);

printf("Enter day        (1-31): ");

scanf("%d", &day);

printf("Enter month      (1-12): ");

scanf("%d", &month);

printf("Enter year       (0-99): ");

scanf("%d", &year);

printf("Enter century    (e.g. 20 for 2026): ");

scanf("%d", &century);

if (!validate_datetime(hour, min, sec, day, month, year, century))

{

printf("\nError: invalid input data.");

printf("\nPress any key...");

getch();

return;

}

wait_rtc_ready();

regB = read_rtc(RTC_STATUS_B);

// set bit 7

write_rtc(RTC_STATUS_B, regB | 0x80);

isBinary = rtc_is_binary_mode();

if (!isBinary)

{

write_rtc(RTC_SECONDS, int_to_bcd(sec));

write_rtc(RTC_MINUTES, int_to_bcd(min));

write_rtc(RTC_HOURS,   int_to_bcd(hour));

write_rtc(RTC_DAY,     int_to_bcd(day));

write_rtc(RTC_MONTH,   int_to_bcd(month));

write_rtc(RTC_YEAR,    int_to_bcd(year));

write_rtc(RTC_CENTURY, int_to_bcd(century));

}

else

{

write_rtc(RTC_SECONDS, (unsigned char)sec);

write_rtc(RTC_MINUTES, (unsigned char)min);

write_rtc(RTC_HOURS,   (unsigned char)hour);

write_rtc(RTC_DAY,     (unsigned char)day);

write_rtc(RTC_MONTH,   (unsigned char)month);

write_rtc(RTC_YEAR,    (unsigned char)year);

write_rtc(RTC_CENTURY, (unsigned char)century);

}

write_rtc(RTC_STATUS_B, regB & 0x7F);

printf("\nNew time has been set successfully.");

printf("\nPress any key...");

getch();

}

// rtc interrupt

unsigned int rate_to_frequency(unsigned char rate)

{

switch (rate)

{

case 0x06: return 1024;

case 0x07: return 512;

case 0x08: return 256;

case 0x09: return 128;

case 0x0A: return 64;

default:   return 1024;

}

}

void set_periodic_rate(unsigned char rate)

{

// rtc periodic frequency set-up

// lower 4 bits regA = RS3..RS0

unsigned char regA;

wait_rtc_ready();

regA = read_rtc(RTC_STATUS_A);

regA = (regA & 0xF0) | (rate & 0x0F);

write_rtc(RTC_STATUS_A, regA);

}

void enable_periodic_interrupt(void)

{

// enable periodic interrupt rtc

// bit 6 rebB = PIE

unsigned char regB = read_rtc(RTC_STATUS_B);

write_rtc(RTC_STATUS_B, regB | 0x40);

}

void disable_periodic_interrupt(void)

{

unsigned char regB = read_rtc(RTC_STATUS_B);

write_rtc(RTC_STATUS_B, regB & 0xBF);

}

void enable_alarm_interrupt(void)

{

// enable alarm rtc interrupt

// bit 5 regB = AIE

unsigned char regB = read_rtc(RTC_STATUS_B);

write_rtc(RTC_STATUS_B, regB | 0x20);

}

void disable_alarm_interrupt(void)

{

unsigned char regB = read_rtc(RTC_STATUS_B);

write_rtc(RTC_STATUS_B, regB & 0xDF);

}

void unmask_rtc_irq(void)

{

// allow irq8 on 2nd pic and irq2 on 1st pic

// otherwise rtc interrupts do not reach program

old_pic1_mask = inp(PIC1_DATA);

old_pic2_mask = inp(PIC2_DATA);

outp(PIC1_DATA, old_pic1_mask & (~0x04));

outp(PIC2_DATA, old_pic2_mask & (~0x01));

}

void restore_pic_masks(void)

{

// recover old pic masks

outp(PIC1_DATA, old_pic1_mask);

outp(PIC2_DATA, old_pic2_mask);

}

void clear_rtc_interrupt_flags(void)

{

// clearing a possible hanging RTC interrupt state

read_rtc(RTC_STATUS_C);

}

void interrupt new_rtc_handler(void)

{

unsigned char regC;

regC = read_rtc(RTC_STATUS_C); // confirm rtc interrupt

if (regC & 0x40) // bit 6 = periodic interrupt

{

rtc_ticks++;

if (rtc_ticks >= target_ticks)

{

delay_done = 1;

}

}

if (regC & 0x20) // bit 5 = alarm

{

alarm_fired = 1;

}

// finish handling irq8:

// firstly 2nd pic, after 1st

outp(PIC2_COMMAND, PIC_EOI);

outp(PIC1_COMMAND, PIC_EOI);

}

void install_rtc_handler(void)

{

// set on handler int 70h

old_rtc_handler = getvect(0x70);

setvect(0x70, new_rtc_handler);

}

void restore_rtc_handler(void)

{

setvect(0x70, old_rtc_handler);

}

void rtc_delay_ms(unsigned long delay_ms, unsigned char rate)

{

// delay func in miliseconds

// delay_ms - time to wait

// rate - frequency code rtc

unsigned int frequency;

unsigned long ticks_needed;

frequency = rate_to_frequency(rate);

ticks_needed = (delay_ms * frequency) / 1000UL;

if (ticks_needed == 0) ticks_needed = 1;

rtc_ticks = 0;

target_ticks = ticks_needed;

delay_done = 0;

clear_rtc_interrupt_flags();

set_periodic_rate(rate);

enable_periodic_interrupt();

// wait till handler set flag

while (!delay_done) {};

disable_periodic_interrupt();

}

void test_delay(void)

{

unsigned long ms;

int rate_choice;

unsigned char rate = 0x06;

clrscr();

printf("========================================\n");

printf("     DELAY VIA PERIODIC INTERRUPTS\n");

printf("========================================\n\n");

printf("Enter delay in milliseconds: ");

scanf("%lu", &ms);

printf("\nSelect RTC interrupt frequency:\n");

printf("1. 1024 Hz (06h)\n");

printf("2.  512 Hz (07h)\n");

printf("3.  256 Hz (08h)\n");

printf("4.  128 Hz (09h)\n");

printf("5.   64 Hz (0Ah)\n");

printf("\nYour choice: ");

scanf("%d", &rate_choice);

switch (rate_choice)

{

case 1: rate = 0x06; break;

case 2: rate = 0x07; break;

case 3: rate = 0x08; break;

case 4: rate = 0x09; break;

case 5: rate = 0x0A; break;

default: rate = 0x06; break;

}

printf("\nWaiting...\n");

rtc_delay_ms(ms, rate);

printf("Delay completed.");

printf("\nPress any key...");

getch();

}

// alarm

int validate_alarm_time(int hour, int min, int sec)

{

if (hour < 0 || hour > 23) return 0;

if (min < 0 || min > 59) return 0;

if (sec < 0 || sec > 59) return 0;

return 1;

}

void set_alarm_time(int hour, int min, int sec)

{

int isBinary;

wait_rtc_ready();

isBinary = rtc_is_binary_mode();

if (!isBinary)

{

write_rtc(RTC_ALARM_SEC,  int_to_bcd(sec));

write_rtc(RTC_ALARM_MIN,  int_to_bcd(min));

write_rtc(RTC_ALARM_HOUR, int_to_bcd(hour));

}

else

{

write_rtc(RTC_ALARM_SEC,  (unsigned char)sec);

write_rtc(RTC_ALARM_MIN,  (unsigned char)min);

write_rtc(RTC_ALARM_HOUR, (unsigned char)hour);

}

}

void test_alarm(void)

{

int hour, min, sec;

clrscr();

printf("========================================\n");

printf("          PROGRAMMABLE ALARM\n");

printf("========================================\n\n");

printf("Enter alarm hour   (0-23): ");

scanf("%d", &hour);

printf("Enter alarm minute (0-59): ");

scanf("%d", &min);

printf("Enter alarm second (0-59): ");

scanf("%d", &sec);

if (!validate_alarm_time(hour, min, sec))

{

printf("\nError: invalid time entered.");

printf("\nPress any key...");

getch();

return;

}

alarm_fired = 0;

set_alarm_time(hour, min, sec);

clear_rtc_interrupt_flags();

enable_alarm_interrupt();

printf("\nAlarm set to %02d:%02d:%02d", hour, min, sec);

printf("\nWaiting for alarm...\n");

while (!alarm_fired) {}; // wait alarm interrupt

disable_alarm_interrupt();

printf("\n*** ALARM TRIGGERED! ***");

printf("\nPress any key...");

getch();

}

void show_menu(void)

{

clrscr();

printf("========================================\n");

printf("      LABORATORY WORK #4 - RTC\n");

printf("========================================\n\n");

printf("1. Read current time and date\n");

printf("2. Set new time and date\n");

printf("3. Delay via RTC periodic interrupts\n");

printf("4. Programmable RTC alarm\n");

printf("0. Exit\n\n");

printf("Your choice: ");

}

void main(void)

{

int choice;

// set self handler and enable irq rtc

install_rtc_handler();

unmask_rtc_irq();

clear_rtc_interrupt_flags();

do

{

show_menu();

scanf("%d", &choice);

switch (choice)

{

case 1:

show_current_time();

break;

case 2:

set_current_time();

break;

case 3:

test_delay();

break;

case 4:

test_alarm();

break;

case 0:

break;

default:

printf("\nInvalid menu option.");

printf("\nPress any key...");

getch();

break;

}

} while (choice != 0);

// disable rtc modes before exit

disable_periodic_interrupt();

disable_alarm_interrupt();

clear_rtc_interrupt_flags();

// restore old handler and old masks pic

restore_rtc_handler();

restore_pic_masks();

}

# Тестирование программы

# Рисунок 4.1 — Вывод текущего времени.

# Рисунок 4.2 — Установка нового времени

# Рисунок 4.3 — Вывод нового времени.

# Рисунок 4.4 — Установка и срабатывание будильника.

# Заключение

В данной лабораторной работе были выполнены все поставленные задачи: написана программа, которая считывает и устанавливает время в часах реального времени, была реализована функция программируемого будильника, используя аппаратное прерывания часов реального времени и режим будильника.

Программа компилировалась в Turbo C++ и запускалась в DOSBox-X